NAMAL UNIVERSITY, MIANWALI

Department of Electrical Engineering

EE-474— Computer Architecture

FINAL PROJECT REPORT

Guided Manual and Implementation Report

Single-Cycle RISC-V SoC with Wishbone Bus
Simulation, GCC Flow, and FPGA LED Deployment

| Student Name | Raheem Bakhsh |
| --- | --- |
| Student ID | NUM-BSEE-2022-10 |
| Course Instructor | Dr. Tassadaq Hussain |
| Submission Date | June 2026 |

Contents

1. CEP Alignment and Completed Work	2

2. System Overview and Architecture	3

3. Project Folder Structure	4

4. Important Files and What Each File Does	5

5. Wishbone Bus Architecture	5

6. GCC-Compatible C Code Flow	6

7. Iverilog and GTKWave Simulation Flow	6

8. FPGA Top Module and LED Connections	7

9. Connecting Colorlight i5 FPGA to WSL Ubuntu	8

10. FPGA Build and Upload Flow	9

11. Complete Combined Command Flow	9

12. Troubleshooting and Common Errors	9

14. Appendix: Final Important Code Snippets	10

14.1 test_program.c LED pattern	10

14.2 fpga_top.v	10

14.3 colorlight_i5.lpf	10

14.4 Final conclusion	11

# 1. CEP Alignment and Completed Work

The CEP requires a single-cycle RISC-V processor in Verilog HDL, RISC-V ISA compatibility, an open-source bus architecture, a GCC-based software flow, simulation using Iverilog/GTKWave, and FPGA prototyping. The updated implementation follows these requirements using a single-cycle RV32I core and a shared Wishbone bus architecture with one master and three memory-mapped slaves.

| CEP Requirement | Implementation in Updated SoC | Status |
| --- | --- | --- |
| Single-cycle RISC-V processor HDL | riscv_core.v contains PC, decode/control, register file, immediate generator, ALU, branch/jump logic, load/store control, and write-back path. | Complete |
| RV32I ISA compatibility | Baseline RV32I operations including R-type, I-type, load/store, branch, jump, LUI and AUIPC paths are supported and tested. | Complete for baseline tests |
| Open-source bus architecture | A shared Wishbone bus connects the RISC-V core master to program memory, data memory, and LED controller slaves. | Complete |
| SoC with program/data/peripheral memory | riscv_soc.v connects CPU master, Wishbone interconnect, program memory slave S0, data memory slave S1, and LED slave S2. | Complete |
| GCC-compatible flow | test_program.c is compiled using the RISC-V GCC toolchain to ELF, converted to binary, and then to program.hex. | Complete |
| Iverilog and GTKWave verification | Iverilog compiles the RTL/testbench; vvp runs simulation; GTKWave displays bus, CPU, memory and LED signals. | Complete |
| FPGA prototyping | Yosys, nextpnr-ecp5, ecppack, and openFPGALoader are used to generate and upload the bitstream to Colorlight i5. | Complete |

# 2. System Overview and Architecture

The system is a small RISC-V SoC. The processor fetches instructions from instruction memory. For data memory and LED output, the CPU uses a Wishbone master interface. The Wishbone address decoder selects the correct slave according to the address range.

| Block | File/module | Purpose |
| --- | --- | --- |
| Top SoC | rtl/riscv_soc.v | Connects CPU, instruction memory, Wishbone interconnect, data memory and LED slave. |
| CPU core | rtl/riscv_core.v | Executes one instruction per clock style path and controls ALU, register file and memory interface. |
| Program counter | program_counter.v / PCplus4.v | Holds current PC and generates PC+4. |
| Control unit | Control_Unit.v | Decodes opcode and generates RegWrite, MemRead, MemWrite, Branch, Jump, MemtoReg, ALUSrc. |
| ALU path | ALU_unit.v / ALU_Control.v | Performs arithmetic, logic, comparison and address calculation. |
| Register file | Register_File.v | Stores RISC-V x0-x31 registers. |
| Immediate generator | immediate_generator.v | Creates I, S, B, U, and J type immediates. |
| Wishbone LSU | wb_master_lsu.v | Converts CPU memory request into Wishbone bus signals. |
| Wishbone decoder | wb_addr_decoder.v | Selects program/data/LED slave from address. |
| Wishbone read mux | wb_read_mux.v | Selects read data from active slave. |
| LED slave | wb_led_slave.v | Memory-mapped LED output register. |
| FPGA wrapper | fpga_top.v | Maps SoC LED output to real FPGA pins. |

# 3. Project Folder Structure

riscv_soc_project_updated/
├── rtl/                         # Synthesizable Verilog RTL files
│   ├── fpga_top.v               # FPGA top wrapper
│   ├── riscv_soc.v              # SoC top
│   ├── riscv_core.v             # CPU core
│   ├── instruction_memory.v     # Direct instruction memory
│   ├── wb_master_lsu.v          # Wishbone master load/store unit
│   ├── wb_addr_decoder.v        # Wishbone address decoder
│   ├── wb_read_mux.v            # Wishbone read-data mux
│   ├── wb_data_mem_slave.v      # Data memory slave
│   ├── wb_prog_mem_slave.v      # Program memory slave
│   └── wb_led_slave.v           # LED MMIO slave
├── sw/                          # C code and conversion script
│   ├── test_program.c           # LED blink / test C code
│   ├── crt0.s                   # Startup code
│   ├── link.ld                  # Linker script
│   └── elf2hex.py               # Binary to program.hex converter
├── sim/                         # Simulation testbenches
│   └── riscv_soc_tb.v
├── build/                       # Generated build files
├── colorlight_i5.lpf            # FPGA pin constraints
├── rtl_compile_order.f          # RTL file list for Iverilog
└── program.hex                  # Generated machine code loaded into instruction memory

# 4. Important Files and What Each File Does

| Order | File | What it does | Why it matters |
| --- | --- | --- | --- |
| 1 | sw/test_program.c | Contains the C software test program and LED pattern code. | This is the program executed by the RISC-V CPU. |
| 2 | sw/crt0.s | Provides startup sequence and calls main. | Required for bare-metal GCC program execution. |
| 3 | sw/link.ld | Places program sections into the SoC memory map. | The reset entry point must match the S0 program memory base address. |
| 4 | sw/elf2hex.py | Converts binary into word-based program.hex. | program.hex is loaded into the Wishbone program memory slave. |
| 5 | rtl/riscv_core.v | Implements CPU datapath, control, and master bus behavior. | The CPU is the only master in the new design. |
| 6 | rtl/riscv_soc.v | Top-level SoC integration file. | This file connects one master to three slaves through the bus. |
| 7 | rtl/wb_prog_mem_slave.v | Program memory slave S0. | Instruction fetch now comes through the shared Wishbone bus. |
| 8 | rtl/wb_data_mem_slave.v | Data memory slave S1. | Used by lw/sw and memory verification tests. |
| 9 | rtl/wb_led_slave.v | LED controller slave S2. | Used for memory-mapped LED output on FPGA. |
| 10 | colorlight_i5.lpf | Maps logical ports to physical FPGA pins. | Wrong pin mapping can make hardware appear non-working. |

# 5. Wishbone Bus Architecture

The SoC uses one shared Wishbone bus. The RISC-V core is the only master. The master sends address, write data, byte select, and control signals to the bus fabric. The address decoder selects exactly one slave for each transaction. The selected slave returns read data and acknowledgement to the master through the read mux and status path.

| Bus Role | Count | Module | Purpose |
| --- | --- | --- | --- |
| Master | 1 | RISC-V core / CPU bus interface | Generates instruction fetch, load/store, and LED MMIO transactions. |
| Slave S0 | 1 | wb_prog_mem_slave.v | Program memory / instruction memory. Selected for address range 0x00000000 to 0x00000FFF. |
| Slave S1 | 1 | wb_data_mem_slave.v | Data RAM. Selected for address range 0x00001000 to 0x00001FFF. |
| Slave S2 | 1 | wb_led_slave.v | Memory-mapped LED controller. Selected for address range 0x00002000 to 0x00002FFF. |

.

| Wishbone Signal | Direction | Meaning in this SoC |
| --- | --- | --- |
| wb_cyc | Master to bus/slave | Indicates that a valid bus cycle is active. |
| wb_stb | Master to bus/slave | Indicates that the current address/data is a valid transfer request. |
| wb_we | Master to bus/slave | Write enable. 1 means write; 0 means read. |
| wb_adr[31:0] | Master to bus/slave | 32-bit memory-mapped address. |
| wb_dat_mosi[31:0] | Master to bus/slave | Write data from CPU to selected slave. |
| wb_dat_miso[31:0] | Selected slave to master | Read data returned from selected slave. |
| wb_sel[3:0] | Master to bus/slave | Byte-lane select for word/byte/halfword access support. |
| wb_ack | Selected slave to master | Acknowledges that the selected slave has completed the transfer. |

| Address Range | Selected Slave | Module | Use |
| --- | --- | --- | --- |
| 0x00000000 - 0x00000FFF | S0 | wb_prog_mem_slave | Instruction fetch / program memory |
| 0x00001000 - 0x00001FFF | S1 | wb_data_mem_slave | Data memory for load/store tests |
| 0x00002000 - 0x00002FFF | S2 | wb_led_slave | LED memory-mapped output |

| Step | Wishbone LED write operation | Expected result |
| --- | --- | --- |
| 1 | C code executes led_write(0x01). | Software wants LED1 ON. |
| 2 | CPU executes store instruction to 0x00002000. | Address belongs to LED slave. |
| 3 | wb_master_lsu drives m_wb_adr=0x00002000 and m_wb_dat_mosi=0x00000001. | Wishbone write transaction starts. |
| 4 | wb_addr_decoder sets sel_s2=1. | LED slave is selected. |
| 5 | wb_led_slave captures write data. | led_out becomes 00000001. |
| 6 | fpga_top maps led_out[4:0] to FPGA pins. | External LEDs show the pattern. |

# 6. GCC-Compatible C Code Flow

This section proves GCC compatibility. The C program is compiled with RISC-V GCC for RV32I, converted to binary, then converted to program.hex. The instruction memory loads program.hex.

| Step | Command | What it does |
| --- | --- | --- |
| 1 | cd ~/final_riscv_soc_project rm -rf build rm -f program.hex riscv_soc.vcd mkdir -p build | Remove previous build files. |
| 2 | riscv64-unknown-elf-gcc \ -Os \ -march=rv32i \ -mabi=ilp32 \ -msmall-data-limit=0 \ -nostdlib \ -nostartfiles \ -ffreestanding \ -fno-builtin \ -DFPGA_BUILD \ -DDELAY_COUNT=5000000 \ -T sw/link.ld \ -o build/test_program.elf \ sw/crt0.s \ sw/test_program.c |  |
| 3 | riscv64-unknown-elf-objcopy \ -O binary \ build/test_program.elf \ build/test_program.bin |  |
| 4 | python3 sw/elf2hex.py build/test_program.bin program.hex |  |

# 7. Iverilog and GTKWave Simulation Flow

| Step | Command | Purpose |
| --- | --- | --- |
| 1 | iverilog -g2012 -o build/sim.out sim/riscv_soc_tb.v -f rtl_compile_order.f | Compile RTL and simulation testbench. |
| 2 | vvp build/sim.out | Run the simulation. |
| 3 | gtkwave riscv_soc.vcd | Open waveform file in GTKWave. |

Expected simulation status after the GCC-compatible test program:

Final LED = 0x55
[PASS] Final LED should be 0x55
[PASS] DMEM[0] ADD result = 8
[PASS] DMEM[1] SUB result = 2
...
Results: 11 PASSED | 0 FAILED
*** PROGRAM.HEX TEST PASSED ***

| GTKWave signal | Where it is found | What to check |
| --- | --- | --- |
| pc_debug | riscv_soc_tb | PC is changing and program is executing. |
| instruction_debug | riscv_soc_tb | Instruction values come from program.hex. |
| alu_result_debug | riscv_soc_tb | ALU and address calculations. |
| dmem0_debug ... dmem50_debug | riscv_soc_tb | Data memory output checks. |
| wb_adr_debug | riscv_soc_tb | Address 0x1000 for data memory and 0x2000 for LED. |
| wb_wdata_debug | riscv_soc_tb | Data being written by CPU. |
| sel_s1_debug / sel_s2_debug | riscv_soc_tb | Correct data memory or LED slave selection. |
| led_out | riscv_soc_tb/DUT | Final LED value or blinking pattern. |

# 8. FPGA Top Module and LED Connections

For FPGA, do not use the simulation testbench. The top module is fpga_top. It instantiates the RISC-V SoC and connects the SoC LED output to real FPGA pins through the LPF constraint file.

| LED | Logical signal | Colorlight i5 pin | Physical connection |
| --- | --- | --- | --- |
| LED1 | led[0] | R1 | R1 → 330Ω resistor → LED anode (+), LED cathode (-) → GND |
| LED2 | led[1] | T1 | T1 → 330Ω resistor → LED anode (+), LED cathode (-) → GND |
| LED3 | led[2] | Y2 | Y2 → 330Ω resistor → LED anode (+), LED cathode (-) → GND |
| LED4 | led[3] | M1 | M1 → 330Ω resistor → LED anode (+), LED cathode (-) → GND |
| LED5 | led[4] | N2 | N2 → 330Ω resistor → LED anode (+), LED cathode (-) → GND |

| Pattern step | C value written to LED_ADDR | 5-bit LED state | Meaning |
| --- | --- | --- | --- |
| 1 | 0x01 | 00001 | LED1 ON, then all OFF |
| 2 | 0x00 | 00000 | All LEDs OFF |
| 3 | 0x02 | 00010 | LED2 ON, then all OFF |
| 4 | 0x00 | 00000 | All LEDs OFF |
| 5 | 0x04 | 00100 | LED3 ON, then all OFF |
| 6 | 0x00 | 00000 | All LEDs OFF |
| 7 | 0x08 | 01000 | LED4 ON, then all OFF |
| 8 | 0x00 | 00000 | All LEDs OFF |
| 9 | 0x10 | 10000 | LED5 ON, then all OFF |
| 10 | 0x00 | 00000 | All LEDs OFF |

# 9. Connecting Colorlight i5 FPGA to WSL Ubuntu

Because the work is done in WSL Ubuntu on Windows, the USB programmer must be shared from Windows into WSL using usbipd. In our case, the programmer appeared as VID:PID 0d28:0204 and was used with openFPGALoader cable option -c cmsisdap.

| Step | Where to run | Command | Purpose |
| --- | --- | --- | --- |
| 1 | Windows PowerShell as Administrator | usbipd list | Show USB devices connected to Windows. |
| 2 | PowerShell Admin | usbipd bind --busid 1-1 | Share the FPGA programmer device. Replace 1-1 with your BUSID. |
| 3 | PowerShell Admin | usbipd attach --wsl --busid 1-1 | Attach the USB device to WSL Ubuntu. |
| 4 | WSL Ubuntu | lsusb | Confirm WSL can see 0d28:0204. |
| 5 | WSL Ubuntu | sudo env "PATH=$PATH" openFPGALoader -c cmsisdap --detect | Detect ECP5 FPGA through CMSIS-DAP. |

Note: If sudo cannot find openFPGALoader, use sudo env "PATH=$PATH" openFPGALoader ... because OSS CAD Suite is installed in the user PATH, not root PATH.

# 10. FPGA Build and Upload Flow

Your actual board was detected as LFE5U-45, so nextpnr must use --45k instead of --25k. A bitstream built for 25K gives an IDCODE mismatch.

| Step | Command | What it does |
| --- | --- | --- |
| 1 | yosys -p "read_verilog rtl/*.v; synth_ecp5 -top fpga_top -json build/soc.json" | Synthesizes Verilog RTL for ECP5 and writes JSON netlist. |
| 2 | nextpnr-ecp5 --45k --package CABGA381 --json build/soc.json --lpf colorlight_i5.lpf --textcfg build/soc.config | Places and routes the design for LFE5U-45 in CABGA381 package. |
| 3 | ecppack build/soc.config build/soc.bit | Converts routed config into FPGA bitstream. |
| 4 | sudo env "PATH=$PATH" openFPGALoader -c cmsisdap build/soc.bit | Uploads bitstream to FPGA through CMSIS-DAP. |

# 11. Complete Combined Command Flow

Paste the following block in WSL Ubuntu after editing test_program.c, fpga_top.v, and colorlight_i5.lpf. It compiles C, generates program.hex, synthesizes, places/routes, packs, and uploads to FPGA.

cd ~/riscv_soc_project_corrected/riscv_soc_project_updated && mkdir -p build && riscv64-unknown-elf-gcc   -Os   -march=rv32i -mabi=ilp32   -nostdlib -nostartfiles   -ffreestanding   -fno-builtin   -T sw/link.ld   -o build/test_program.elf   sw/crt0.s sw/test_program.c && riscv64-unknown-elf-objcopy -O binary build/test_program.elf build/test_program.bin && python3 sw/elf2hex.py build/test_program.bin program.hex && yosys -p "read_verilog rtl/*.v; synth_ecp5 -top fpga_top -json build/soc.json" && nextpnr-ecp5   --45k   --package CABGA381   --json build/soc.json   --lpf colorlight_i5.lpf   --textcfg build/soc.config && ecppack build/soc.config build/soc.bit && sudo env "PATH=$PATH" openFPGALoader -c cmsisdap build/soc.bit

# 12. Troubleshooting and Common Errors

| Error / symptom | Cause | Correction |
| --- | --- | --- |
| lsusb shows nothing in WSL | USB device not passed from Windows to WSL. | Use usbipd list, bind, and attach from PowerShell Admin. |
| openFPGALoader: No device found | WSL cannot see the programmer or wrong cable option. | Confirm lsusb shows 0d28:0204 and use -c cmsisdap. |
| sudo: openFPGALoader command not found | sudo does not inherit OSS CAD Suite PATH. | Use sudo env "PATH=$PATH" openFPGALoader ... |
| IDCODE mismatch | Bitstream built for wrong ECP5 size. | Use nextpnr-ecp5 --45k for detected LFE5U-45. |
| Failed to open build/soc.config | nextpnr did not generate soc.config. | Run nextpnr first and check for errors. |
| Conflicting initialization values for mem[0] | Memory initialized by for loop and $readmemh together. | Remove for-loop initialization; keep only $readmemh when program.hex is padded. |
| LED always ON | LED may be active-low, wrong pin, or C pattern has no OFF interval. | Use led_write(0x00) between each LED pattern and try inverted led assignment. |
| Simulation passes but FPGA LED does not blink | Likely pin mapping or physical LED connection issue. | Check LPF pins, resistor, LED polarity, GND, and active-low inversion. |
| GCC program fails in simulation | Unsupported instructions such as lbu/sb may be generated. | Run objdump grep and simplify C code to word-only operations or implement byte instructions. |

# 14. Appendix: Final Important Code Snippets

## 14.1 test_program.c LED pattern

#define LED_ADDR ((volatile unsigned int *)0x00002000)
#define DELAY_COUNT 5000000

static void led_write(unsigned int pattern)
{
    *LED_ADDR = pattern;
}

static void delay_1s(void)
{
    volatile unsigned int i;
    for (i = 0; i < DELAY_COUNT; i++) {
    }
}

void main(void)
{
    while (1) {
        led_write(0x01); delay_1s(); led_write(0x00); delay_1s();
        led_write(0x02); delay_1s(); led_write(0x00); delay_1s();
        led_write(0x04); delay_1s(); led_write(0x00); delay_1s();
        led_write(0x08); delay_1s(); led_write(0x00); delay_1s();
        led_write(0x10); delay_1s(); led_write(0x00); delay_1s();
    }
}

## 14.2 fpga_top.v

`timescale 1ns/1ps

module fpga_top (
    input  wire       clk,
    output wire [4:0] led
);

    wire reset;
    wire [7:0] led_bus;

    assign reset = 1'b0;

    riscv_soc SOC (
        .clk(clk),
        .reset(reset),
        .led_out(led_bus)
    );

    assign led[0] = led_bus[0];
    assign led[1] = led_bus[1];
    assign led[2] = led_bus[2];
    assign led[3] = led_bus[3];
    assign led[4] = led_bus[4];

endmodule

## 14.3 colorlight_i5.lpf

LOCATE COMP "clk" SITE "P3";
IOBUF PORT "clk" IO_TYPE=LVCMOS33;

LOCATE COMP "led[0]" SITE "R1";
IOBUF PORT "led[0]" IO_TYPE=LVCMOS33;

LOCATE COMP "led[1]" SITE "T1";
IOBUF PORT "led[1]" IO_TYPE=LVCMOS33;

LOCATE COMP "led[2]" SITE "Y2";
IOBUF PORT "led[2]" IO_TYPE=LVCMOS33;

LOCATE COMP "led[3]" SITE "M1";
IOBUF PORT "led[3]" IO_TYPE=LVCMOS33;

LOCATE COMP "led[4]" SITE "N2";
IOBUF PORT "led[4]" IO_TYPE=LVCMOS33;

## 14.4 Final conclusion

The project successfully implements a single-cycle RISC-V SoC in Verilog HDL with an open-source Wishbone bus architecture. The SoC includes instruction memory, data memory, and an LED memory-mapped slave. A GCC-generated program.hex file is loaded into the instruction memory and executed by the RISC-V core. The design was verified in Iverilog/GTKWave and synthesized for the Colorlight i5 ECP5 FPGA using Yosys, nextpnr-ecp5, ecppack, and openFPGALoader. The hardware LED pattern confirms that the C program controls FPGA LEDs through the RISC-V CPU and Wishbone LED slave.